JECRC University, Jaipur

Department of Physics

B. Tech Ist Year – Ist Semester July-Dec 2025-26

Applied Physics (DPH001B)

Assignment-III

- Very Short Answer Questions 							         (10)

- Name the charge carriers which are responsible for flow of currents in conductors and intrinsic semiconductors?                                                                                                (2)

- Define the term mobility. Does electron and hole have same mobility?                         (2)

- Draw the symbol of PN junction diode, Zener Diode, Varactor Diode, LED and Photodiode.                                                                                                                      (2)

- What is the use of Varactor diode in circuit? Also mention how Varactor diode is connected in forward bias or reverse bias condition?                                                     (2)

- What is a photodiode how it is different from LED?                                                      (2)

- Answer the following briefly							         (21)

- Find the resistivity of an intrinsic semiconductor with carrier concentration 2.5x1019/m3. The electron and hole motilities are 0.40 m2/V-s and 0.20 m2/V-s respectively.             (7)

- What is a Zener diode? Explain the breakdown mechanism in Zener diode. How Zener breakdown different from avalanche breakdown.                                                           (7)

- For Intrinsic semiconductors if me=mh prove that the Fermi energy level lies exactly in the middle of highest energy Ev of valance band and lowest energy Ec of the conduction band.                                                                                                                                (7)

- Answer the followings explicitly.						                     (33)

- Derive the expression for electron concentration and hole concentration starting from expression of density of states and Fermi function. Also find the total carrier concentration for intrinsic semiconductor.                                                                    (11)

- Derive the expression for conductivity of intrinsic and extrinsic semiconductors.                                                                                                 Explain the effect of concentration of minority charge carrier due to addition of impurity in intrinsic semiconductors. (11)

- Draw a circuit diagram to draw the volt Ampere (V-I) characteristic of PN junction diode. Also draw probable V-I characteristics for PN junction diode and explain different knee voltage and breakdown voltage using the diagram.                                              (11)